================================================================================

Airfoil generator utilizing the Joukowski transformation

Written by:  Russell Leighton

762 1/2 W. Newgrove

Lancaster, CA  93534

22 March 1987

================================================================================

The following paper is a condensed version of the paper I

originally wrote describing the Joukowski tranformation.  It was

submitted to the Aeronautical and Mechanical Engineering

Department, School of Engineering and Technology of the

California Polytechnic State University in San Luis Obispo, CA

June 1984.

================================================================================

A Computational Flow Visualization Technique Utilizing the

Joukowski Transformation

Written by:  Russell A. Leighton

LIST OF SYMBOLS:

u - Real component in w-plane

v - Imaginary component in w-plane

w - Complex resultant plane

x - Real component in z-plane

y - Imaginary component in z-plane

z - Complex source plane

i - Imaginary unit (square root of -1)

INTRODUCTION:

Conformal mapping is a very useful mathematical tool and has

applications in the engineering field.  One particular application

utilizes conformal mapping to simplify the visualization of fluid

flow about airfoil sections.  By simplfing the mathematics this

technique allows for faster computation and therefore could be used

for real time computational flow visualization.  The potential uses

for a computational flow visualization technique range from an

educational tool, illustrating the behavior of fluid flow about

airfoils, to an advanced modeling tool.  Since the actual flow is

computed about a simple shape, the circle, development of the flow

visualization equations is straight forward.

The following sections will discuss the development of the mapping

equations, the streamline and pressure distribution equations, and

reverse mapping equations necessary for the calculation of the

circle parameters given airfoil data.

DISCUSSION:

Conformal Mapping

-----------------

Conformal mapping is a mathematical tool that can be used to

visualize the nature of complex functions.  A definition of

conformal mapping can be understood by picturing two distinct

planes, the source plane (z-plane) and the resultant plane

(w-plane).  Given a domain D of the z-plane and a complex function,

w = f(z) relating the z-plane to the w-plane, for each point in

domain D there exists a corresponding point in the w-plane.  If the

function, f(z) is an analytic function then the mapping given by

f(z) is said to be conformal, or angle-preserving, except at points

where the derivative, f'(z) is zero.

The general form of the complex function relating the z-plane to

the w-plane is:

(1)		w = f(z) = u(x,y) + (i)v(x,y)

where:  z = x + (i)y

The point wo = f(zo) corresponding to a point zo is called the

"image" of the point zo with respect to the mapping defined by

f(z).  A set of points representing a function in the z-plane will

have a corresponding set of points, or "image" in the w-plane.

Points located in the z-plane, such that the derivative of the

mapping function goes to zero, are called critical points.  At

these points the mapping is said to be non-conformal (i.e. the

angles are not preserved).  As will be shown, these points are

important for the following mapping.

The Joukowski Transformation

----------------------------

The following mapping function is important in the field of

aerodynamics because of the nature of its' transformation.  With

this mapping function if a circle is plotted in the z-plane, such

that its' center is near the origin and it passes through one

critical point, it will be transformed into an airfoil shape. The

form of this function is:

(2)		w = z + 1/z

Its' derivative is:

(3)		w' = 1 - 1/z = (z + 1)(z - 1)/z

Therefore, the mapping will be conformal except at points z = 1

and z = -1, where w' goes to zero.  If plotted it would be evident

that passing through one of these points will produce a sharp edge

resembling the trailing edge of an airfoil.  If the geometry of a

circle is such that one of the critical points is intersected

while the other is bypassed, an airfoil shape will result from the

transformation.  This transformation is commonly known as the

Joukowski transformation which was named for the Russian

mathematician, Nikolai Jegorovich Joukowski for his initial use

of this mapping function.

Computer Implementation

-----------------------

The derivation of the equations suitable for computer implementation

is as follows.  Given the complex function:

(4)		w = u + (i)v = z + 1/z

where u is the horizontal component in the w-plane and v is the

vertical component.  If

(5)		z = x + (i)y

then

(6)		1/z = [1/(x + (i)y)][(x - (i)y)/(x - (i)y)]

Separate the real and imaginary parts to obtain

(7)		1/z = (x/s) - (i)(y/s)

where:  s = x^2 + y^2

therefore, from equation (4)

(8)		u = x + x/s

(9)		v = y - y/s

where:  s = x^2 + y^2

These equations define the mapping process and can be easily

implemented into computer software (see C source listing).

To define the circle in the z-plane the radius and the location

of its' center are necessary.  Since the circle must pass through

one of the two critical points and bypass the other it is

necessary that the radius be greater than one.  This is actually

more information than is required to define the circle.  For

example, one component of the circle center location could be

calculated from the other component, the radius, and the known

critical point (e.g. -1,0).  Likewise, any of the other parameters

may be calculated if the remaining parameters are known.

The Inverse Mapping

-------------------

It has been shown that airfoil shapes may be easily obtained from

the Joukowski transformation of the relativily simple shape, the

circle.  However, it is not convenient to define these airfoil

shapes in terms of their corresponding circle parameters (the radius

and center location).  To determine the necessary circle parameters,

an inverse mapping (or a mapping from the w-plane to the z-plane)

may be performed.

Two airfoil parameters, the camber and thickness, are useful for

defining the airfoil.  A very simple inverse mapping, requiring

only three points to be mapped, can be found by specifing the

camber and thickness at the mid-chord location.  The derivation

of this inverse mapping is rather involved, therefore, it is

left to the reader to determine, if so interested (or just take a

look at the C source listing and try to figure it out).

Flow About Cylinders and Airfoils

---------------------------------

The usefulness of the Joukowski transformation is derived mostly

from the fact that a circle is a much simpler shape than the

airfoil section.  This property of this particular mapping can be

further exploited by recognizing that not only is the airfoil

exactly represented by the circle (or a unit depth cylinder), but

the region about the airfoil is also represented by the region

surrounding the cylinder.  This means that any curves plotted about

the cylinder, in the z-plane, have corresponding curves located

about the airfoil, in the w-plane.  Specifically, streamline and

pressure distribution plots may be computed for the cylinder and

then mapped onto the w-plane in order to obtain the corresponding

streamline and pressure distribution plots about the airfoil.

Again the equations for the streamline and pressure distribution

plots can be derived by the reader if so interested (the theory

can be found in most aeronautical engineering references).

Angle of Attack and Rotation Tranformation

------------------------------------------

The angle of attack may be included in the equations describing

the flow about the cylinder.  It is interesting to note that any

changes in angle of attack will not result in any change in the

flow about the cylinder except that the angle at which the flow

enters the region about the cylinder should be equal to the

negative value of the angle of attack.  A simple rotation

transformation would bring the flow direction back to the

horizontal, resulting in no apparent change from a zero angle of

attack.  It should be noted, however, that the local coordinate

axis is no longer coincident with the global coordinate axis.

Because of this difference the Joukowski transformation will

result in an airfoil at an angle of attack with the flow direction

coming into and leaving the region of the airfoil, parallel to

the horizontal global coordinate axis.

The same is also true for the pressure distribution.  At any given

angle of attack, the pressure distribution will remain the same

for the cylinder.  However, once transformed to the w-plane, the

resulting pressure distribution will be about an airfoil at the

given angle of attack.

Possible Additions to the Model

-------------------------------

The equations for the streamline plot and the pressure distribution

are easily derived for flow about a simple cylinder.  The equations,

or model used in the program assume invisid, irrotational flow and

were therefore the simplest to derive.  A possible addition to this

model would be to incorporate boundary layer effects into the

equations describing the flow about the cylinder.

Another addition, that is important if precise airfoil geometry is

required, is the incorporation of a complete inverse mapping

capability.  A complete inverse mapping would allow for a point by

point description of the airfoil as input to the model.  This

airfoil geometry would, in turn, be mapped from the w-plane onto

the z-plane resulting in an approximate cylindrical shape.  The flow

model may then be developed for this approximate cylinder and the

corresponding flow model, describing the flow about the airfoil,

may then be obtained by the forward mapping process.  Although

complex this addition would increase the accuracy of this modeling

technique giving results suitable for comparison to experimental

results.

CONCLUSION:

By simplifing the modeling process, conformal mapping and in

particular, the Joukowski transformation, offers a simple and

fast method for computational flow visualization of fluid flow

about arbitrary airfoil sections.  The equations necessary for the

mapping process are readily incorporated into a computer program

which aids in the production of a graphical output of the

transformation.

The potential use of the Joukowski transformation is only limited

by the fluid model developed to describe the flow about the

cylinder.  Since the modeling process is simplified, complex

fluid models can be more easily incorporated.